# SOP-E024

Determination of pH Using an Orion Research EA 920 Expandable ionAnalyzer

Objective: To determine the pH of an aqueous solution using an Orion Research EA 920 Expandable ionAnalyzer

Procedure:

1.  Verify that the Orion Research EA 920 Expandable ionAnalyzer is in status 2, pH mode, and sample function. The instrument is located in the Hansen building, room B050.

2a.  Remove the pH electrode from the storage solution.

2b.  Rinse the electrode with Millipore filtered water and gently shake off excess.

3a.  Insert the electrode into the solution so that the reference junction is immersed.

3b.  Stir the solution while measuring to assure homogeneity.

4.  When the reading has stabilized, the READY message will appear. The solution pH can then be read from the display.

5a.  Rinse the electrode with Millipore filtered water and gently shake off excess.

5b.  Return the electrode to the storage solution.

-------------------------------------------------------------------------------------------------------------------------------

Created by: Gretchen Lawler

Verified by:			 	Date:

Print Name

Sign Name