# SOP-E050  (Larisa)

AMGEN Automation of HL60 Assay

Cell Culture:

1.  HL60 cells - log cells are bulk frozen and stored

2.  Prewarm  37 deg buffers (TCPBS in naopure water), plus 0.1% BSA final), 0.2um filters

3.  50 ml conical tube of cells 1 x 10E7/ml thawed to 50% thaw, add equal volume of RPMI to accelerate thaw,

place into 37 deg incubator

1.  Medium from 4 deg to 37 deg 10 min

2.  Cell counted usually Ok

3.  Sanitize robot and hood (15 minutes)

Medium: DMEM

Automation:

1.  Format for 384 well plate is set:

2.  Columns ##2-20, chemical compounds at 9 concentrations, uM,: 300, 100, 33.33, 11.11, 3.70, 1.23, 0.41, 0.137, 0.046.

3.  Columns 1,2,21-24, controls: no compounds,.

4.  2 dye panels, panel 1: rows  A,C,E, G, I, K, M, O, panel 2 – rows B, D, F, H, J, L, N, P

5.  Duplicates per each dye,

6.  Volumes: cells 20ul of 1 x 10E7 (20,000 cells total per well)

7.  Chemicals: 20 ul of 2x conc

Assay:

Biomek FX robot

1. Chemicals prepared same day as assay - in 96 well plate, 3 fold serial dilutions in medium                                                           (RMPI-1640 + 10 % FBSc), 100 ul of 2xconcentrations of chemicals.

- Transfer chemicals to 384-well plate (20 ul/well).

Biomek300 (inside Biosafety hood)

- Add cells (20 ul per well).

- Incubate 6 hours at 37C, CO2 incubator.

- Spin down plate 250 x g 5 min.

- Aspirate supernatant - ~30 ul removed of total of 40 (75% removal).

- Add 40ul dye Viability/Redox panel to rows A,C,E, G, I, K, M, O;

- Add 40ul dye JC1  to rows B, D, F, H, J, L, N, P; Valinomycin is added to last 4 wells for JC-1(welLs P21-P24).

- Cover with tape and spin plate 250g x 30 seconds (to remove bubbles).

- Shake 10 seconds @ 2200 RPM.

- Sent to flow ( to place to incubator at 37C for 10 min)

Dyes are made in PBS in 0.1% BSA  Calcein 30,000 dilution of 1 mg/ml, final 0.03ug/ml ; mBBr , stock 40mM, final 40 uM,  Mito-sox  10mM in DMSO, final 10uM.

JC-1 stock 5mM, final 5 uM, valinomycin 1000 dilution of stock.

Cells and dyes in bulk given to flow for instrument setup

Flow Cytometry:

Instrument must be on 30 mins prior to receiving samples

Incubate plates 10 min @ 37 deg (for dye incubation)

Bulk cells used to set up cytometer:

2 ml for JC-1 dyed - split ??????

Cells - 1 hour prior to end of 6 hour incubation flow will do setup controls

Flow Cytometer setup and operation:

- Set up flow protocols

The protocol is shown in Appendix / JC1 / Redox

Hypercyte setup and operation:

Set up prewritten procols:

JC1

Redox

30 second prime, 30 second shake @3000 RPM, run

1.  Run REDOX first (rows a, c, etc) / Save and covert data to Hypercyte

2.  Wash probe 1 minute, rinse 1 minutes

3.  Run JC1 protocol / Save and covert data to Hypercyte

4.  Wash probe 1 minute, rinse 1 minutes

Data Processing & Archiving:

Run hypercyte analysis

-------------------------------------------

Created by: Larisa Avramova

Verified by:			 	                                     Date:  	7/11/2012

Print Name

Sign Name