# SOP-P028

#### Preparation of Stock 1 M Tris

Objective: To prepare a stock solution of Tris for use in preparation of HBSS buffers

Procedure:

1.  Measure out 400 ml Millipore filtered water in a graduated cylinder.

2.  Pour the water into a clean beaker containing a stir bar.

3.  While stirring gently using a magnetic stirplate, add the following to the beaker:

8.0 g Tris base (tris[hydroxymethyl]aminomethane; Sigma #T-1503)

68.5 g Tris HCl (tris[hydroxymethyl]aminomethane hydrochloride; Sigma #T-3253)

4.  Stir until dissolved.

5a. Continue to stir and determine the pH of the solution using an Orion Research EA920 Expandable Ion Analyzer (refer to SOP#-P024 for Determination of pH Using an Orion Research EA920 Expandable Ion Analyzer). The analyzer is located in the Hansen building, room B050.

5b. Adjust the pH of the solution to 7.3. Using a 5.25" Pasteur pipette fitted with a rubber pipette bulb, add drop wise 1 N hydrochloric acid (HCl) if the pH is higher than 7.3, or 1 N sodium hydroxide (NaOH) if the pH is lower than 7.3, until the correct pH is obtained.

6.  Remove the stir bar and pour the solution into a graduated cylinder.

7.  Add Millipore filtered water to the solution until the volume reaches 500 ml.

8.  Pour the 1 M Tris into a container such as a glass bottle with cap.

8.  Label the bottle with content information, date, and your initials.

9.  Store refrigerated at 0-5°C. Expiration date is six months from date of preparation.

-------------------------------------------------------------------------------------------------------------------------------

Created by: Gretchen Lawler

Verified by:			 	Date:

Print Name

Sign Name